Central Processing Unit Architecture - Final Project

Submitted by:

Yanai Mason – 206701591

Roy Shani – 315995852

MIPS based MCU Architecture

Contents

Overall system description	4

MCU	4

MIPS	7

Instruction Fetch	8

Instruction Decode	9

Execute	10

Data Memory	11

Basic Timer	12

Interrupt controller	14

GPIO	16

Critical path	18

Data analysis	20

ModelSim Results	20

Signal tap results	22

TABLE OF FIGURES

Figure 1 - BLOCK DIAGRAM OF SYSTEM	4

Figure 2 - RTL of MCU	4

Figure 3- Logic usage per component	5

Figure 4 - Port table	6

Figure 5 - MIPS diagram	7

Figure 6 - MIPS RTL	7

Figure 7 - Ifetch diagram	8

Figure 8 - Idecode diagram	9

Figure 9 - execute diagram	10

Figure 10 - Dmemory diagram	11

Figure 11 - Basic timer diagram	12

Figure 12 - Basic timer RTL	13

Figure 13 - Interrupt controller diagram	14

Figure 14 - interrupt controller RTL	15

Figure 15 - GPIO diagram	16

Figure 16 - GPIO RTL	17

Figure 17 - calculated Fmax	18

Figure 18 - Critical Path tech view	18

Figure 19 - critical path chip planner view	19

Figure 20 - maximum path description	19

Figure 21- min path from hold slack	19

Figure 22- wave diagram test 1 interrupt based	20

Figure 23 - wave diagram test 2 interrupt based	21

Figure 24 - signal tap test 2 interrupt based	22

# Overall system description

## MCU

In this project, we want to build an MCU (Microcontroller Unit) processor based on a 5-stage MIPS Single-Cycle architecture: IF, ID, EX, MEM, WB. We have been asked to add additional hardware components (Basic Timer, FIR, GPIO, Interrupt Controller) that will work together as needed. The communication between the different hardware components will be via 3 bus lines that will transfer relevant information between the producers and consumers. Below is a block diagram:

Figure 1 - BLOCK DIAGRAM OF SYSTEM

The RTL diagram of the MCU is below:

Figure 2 - RTL of MCU

The logic usage for each block is given by:

Figure 3- Logic usage per component

The port table is given by:

Figure 4 - Port table

## MIPS

This component is the CPU of the MCU. It is a Single-Cycle architecture and supports the instructions provided in the exercise. It can execute any program that uses only these instructions. Below is the processor with the port names:

Figure 5 - MIPS diagram

Below is the RTL of the MIPS processor:

Figure 6 - MIPS RTL

## Instruction Fetch

The Instruction Fetch (Ifetch) module is the core component of a single-cycle MIPS processor responsible for providing the Program Counter (PC) and fetching instructions from memory. It contains a PC register that is updated on each clock cycle based on a multiplexer's output, which selects the next address from four possible sources: a sequential increment (PC_plus_4), a conditional branch address, a jump target address, or an interrupt service routine address. The module utilizes an internal ROM, instantiated with an altsyncram component, to store the program's instructions. he Ifetch module also includes logic for branch and jump instructions, using control signals like Branch, Zero, and jump to determine the correct next PC, ensuring proper program flow. Additionally, it provides outputs for the current PC and the PC+4 address, which are used by other modules in the processor. Below is its diagram:

Figure 7 - Ifetch diagram

## Instruction Decode

The Instruction Decode (Idecode) module serves as the processor's decode and register file stage, primarily responsible for reading instruction fields, accessing the register file for operand values, and preparing data for subsequent pipeline stages. This module contains an array of 32 general-purpose registers that act as the processor's register file. It takes a 32-bit instruction as input and extracts the source and destination register addresses, using these to read two operands from the register file. A key function of the module is to handle the sign extension of 16-bit immediate values from instructions to 32 bits. The module also includes a multiplexer to select the correct data to write back to the register file (from the ALU, memory, or a special value for a Jump-and-Link instruction), as well as specific logic to manage special-purpose registers for interrupt handling and to calculate jump addresses for control flow instructions. The commonly seen write back module is implemented within this component. Below is idecode diagram:

Figure 8 - Idecode diagram

## Execute

The Execute module is the stage of the MIPS processor responsible for performing arithmetic and logic operations, as well as calculating branch addresses. It includes a core Arithmetic Logic Unit that takes two 32-bit operands (Ainput and Binput) and a 3-bit control signal (ALU_ctl) to perform various operations, including addition, subtraction, AND, OR, XOR, and set-less-than. The module's internal logic uses a multiplexer to select the second operand (Binput) for the ALU, which can either be a value from the register file or a sign-extended immediate value. Additionally, the module calculates branch addresses by adding the sign-extended immediate value to the current Program Counter (PC) and generates a Zero flag that indicates if the ALU result is all zeros, which is essential for conditional branch instructions. The module also contains specific logic to handle instructions like shifts and multiplication. Below is its diagram:

Figure 9 - execute diagram

## Data Memory

The dmemory module implements the main data storage for the MIPS processor, functioning as a single-port RAM for reading and writing 32-bit data. It primarily uses an altsyncram component, which is configured to allow both read and write operations, utilizing the clock for synchronization. The module calculates the memory address (dMemAddr) based on the input address from the ALU, with specific logic to adapt the address width for either ModelSim simulation or FPGA hardware implementation. Write operations are controlled by the Memwrite and ena signals, asserting wren_a to enable data storage, while read data is continuously available on the read_data output based on the provided address. This module is crucial for load and store instructions, allowing the processor to interact with its main data storage. Below is its diagram:

Figure 10 - Dmemory diagram

## Basic Timer

The Basic Timer (BTIMER) module is a versatile hardware component that provides timing functionality, interrupt generation, and Pulse-Width Modulation (PWM) capabilities. The module's behavior is controlled by several dedicated registers, including BTCTL (Control Register), BTCCR0 and BTCCR1 (Compare Registers), and a counter register BTCNT. It features a configurable clock divider that generates an internal clock (CLK) from the master clock (MCLK) based on the BTSSEL field in BTCTL. The BTCNT counter increments on each divided clock cycle, and its value can be cleared by a strobe (BTCLR) or held (BTHOLD). When BTCNT matches either BTCCR0 or BTCCR1, an event can be triggered. For PWM, the BTOUT signal is toggled when the counter reaches a compare value, allowing for duty cycle control. It also generates an interrupt flag (BTIFG) based on a selected bit from the counter or when the counter matches BTCCR0, allowing the processor to handle time-based events. The BTCNT value can be read or written to through a dedicated address on the bus, allowing the processor to interact with the timer's state. Below is its diagram:

Figure 11 - Basic timer diagram

And the basic timer’s RTL:

Figure 12 - Basic timer RTL

## Interrupt controller

The interrupt module acts as the central orchestrator for all interrupt requests in the MIPS system, managing multiple interrupt sources, prioritizing them, and generating the main interrupt signal to the CPU. It features an Interrupt Flag Register (IFG) that stores the status of all active interrupt requests and an Interrupt Enable Register (IntrEn) that allows software to enable or disable individual interrupt sources. The module continuously monitors various external IntrSrc inputs (like timers and key presses) and, when an enabled interrupt becomes active, asserts the main INTR signal to the CPU (provided the Global Interrupt Enable GIE is also active). Upon receiving an interrupt acknowledge signal (INTA) from the CPU, the module clears the corresponding interrupt flag and provides an IRQ_OUT vector to indicate which interrupt source is currently active. Furthermore, it determines an TypeReg (Interrupt Vector Address) value based on the highest-priority active interrupt, which the CPU uses to jump to the correct Interrupt Service Routine. The module also allows the CPU to read and write to its internal registers (like IFG and IntrEn) via the data and address buses, enabling dynamic control over interrupt behavior. Below is its diagram:

Figure 13 - Interrupt controller diagram

And its RTL:

Figure 14 - interrupt controller RTL

## GPIO

The GPIO module serves as a bridge between the processor's bus and a set of external peripherals, allowing the CPU to read from switches and write to LEDs and seven-segment displays. The module is primarily a structural design, instantiating several instances of two sub-modules: OutputPeripheral for the LEDs and seven-segment displays and InputPeripheral for the switches. Each peripheral is individually controlled by a dedicated chip select signal (CS_LEDR, CS_SW, etc.), ensuring that data transfers on the DataBus are directed to the correct component. When the processor performs a write operation with a valid chip select, the OutputPeripheral modules capture the data from the DataBus and drive the corresponding external output pins. Conversely, when the processor performs a read operation, the InputPeripheral module places the current state of the switches onto the DataBus, allowing the CPU to read the input values. Below is its module:

Figure 15 - GPIO diagram

The RTL of this component is below:

Figure 16 - GPIO RTL

# Critical path

The critical path is the longest path of logic propagation within the digital circuit we built. It represents the slowest path in the system, and as such, we need to use it to determine the maximum achievable clock frequency for the MCU's design. The critical path in this project is:

Figure 17 - calculated Fmax

The critical path in tech view is:

Figure 18 - Critical Path tech view

The critical path in chip planner is:

Figure 19 - critical path chip planner view

The critical path is:

Figure 20 - maximum path description

Minimum path is:

Figure 21- min path from hold slack

# Data analysis

This section of the report will discuss obtained results in terms of timing analysis, simulation, wave diagrams and signal tap.

## ModelSim Results

ModelSim Wave diagram analysis:

Figure 22- wave diagram test 1 interrupt based

The diagram shows the interrupt reaction to user key press on The user's input key2 press triggers an interrupt request. The interrupt controller detects this and sets the corresponding bit in the Interrupt Flag Register (IFG) to ‘0010000’, as indicated in the wave diagram. Since the Global Interrupt Enable (GIE) bit is set by the ori $k0,$k0,0x01 instruction, the INTR_active signal is asserted. The processor then saves its current state and jumps to the KEY2_ISR based on the interrupt vector table (IV). The state of the switches is written into register $t0. The value in $t0 is then written to PORT_HEX2, PORT_HEX3, and PORT_LEDR, causing the corresponding seven-segment displays and the LEDs to update. As seen in the wave diagram, the LEDR output changes from 0x01 to 0x02.

Figure 23 - wave diagram test 2 interrupt based

Before the interrupts occurs, the timer is set up by the main routine. The BTCCR0 is loaded with 0x010 (changed from 3125 to 16 to be able to observe behaviour on modelsim), and BTCCR1 is loaded with half of that value. The BTCTL register is configured to set BTSSEL to 3, which corresponds to a DIV value of 1000. The BTOUTEN bit is set to 1, enabling the PWM output, and the Global Interrupt Enable (GIE) is set. Pressing KEY3 triggers an interrupt. The interrupt controller detects this, sets the KEY3 flag in the IFG register ‘00100000’, and since GIE is active, the INTR_active signal is asserted. The processor then jumps to the KEY3_ISR based on the interrupt vector table. The KEY3_ISR first changes the timer's configuration. It loads BTOUTEN_BTSSEL0 into the BTCTL register, changing the BTSSEL value from ‘11’ to ‘00’. This alters the clock divider (DIV) from ‘1000’  to ‘0001’, as shown in the provided waveform. Consequently, the PWM and interrupt frequencies are changed from MCLK/(8*16) to MCLK/16.

## Signal tap results

The signal tap test was executed and results are displayed below:

Figure 24 - signal tap test 2 interrupt based

As can be seen, when performing test 2 of the interrupt based IO, with the alteration of the frequency form being divided by 3125 to 16, pressing key3 triggers interrupt, as can be seen in the IFG buffer holding the value of 0x10h. MIPS:CPU|PC[9..0] shows a short excursion to the KEY3 ISR address range and then a jr $k1 back to the main loop. This confirms the core actually serviced the interrupt (not just latched IFG). Inside KEY3_ISR your code does lw IFG; andi, CLRKEY3IFG; sw IFG. In the waveform the IFG field drops back to 00h a few cycles after the PC hits the ISR, confirming the flag was cleared in software. Immediately after the ISR return, the BTIMER:Basic_Timer|PWM waveform shows a shorter period, and BTOUT edges space closer together.